Copyright 2020 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

This Product MasterSpec Section is licensed by Deltek, Inc. to Kolbe Windows & Doors ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Kolbe Windows & Doors, 1323 S. 11th Ave., Wausau, WI 54401; Phone (715) 842-5666; Website: www.kolbewindows.com; Email: professionals@kolbewindows.com.

For information about MasterSpec, contact Deltek at (800) 424-5080 or visit masterspec.com.

SECTION 083519.13 - MULTIPANEL FOLDING WOOD-FRAMED GLASS DOORS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at ProductMasterSpec.com>

Access BIM Content on BIMsmith Market:

<Double click here to view products on BIMsmith Market>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

MasterSpec includes provisions for LEED v4, LEED v4.1, IgCC, and Green Globes.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Multipanel folding wood-framed glass doors.

          - Multipanel folding aluminum-clad-wood-framed glass doors.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 083219.13 "Multipanel Sliding Wood-Framed Glass Doors" for multipanel wood-framed glass doors that operate by sliding.

          - Section 085200 "Wood Windows" for related wood-framed transom and sidelite windows and mullions and for coordinating combinations and finish among wood fenestration units.

          - [Section 087100 "Door Hardware"] [Section 087111 "Door Hardware (Descriptive Specification)"] for hardware not specified in this Section.

          - Section 099300 "Staining and Transparent Finishing" for on-site finishing of bare, unfinished, and factory-primed multipanel folding wood-framed glass doors.

          - [Section 099113 "Exterior Painting"] [and] [Section 099123 "Interior Painting"] for on-site finishing of bare, [unfinished] [and] [factory-primed] multipanel folding wood-framed glass doors.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for multipanel folding wood-framed glass doors.

        - Shop Drawings:

          - Include plans, elevations, sections, and installation details.

          - Indicate dimensions, configuration of panels, and stacking layout.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" or "Samples for Verification" Paragraph, or both.

        - Samples: For each multipanel folding wood-framed glass door and for each color specified; 12-inch- (300-mm-) long section with factory-applied color finish.

        - Samples for Initial Selection: For doors and hardware with factory-applied color finish.

        - Samples for Verification: For multipanel folding wood-framed glass doors and components required, prepared on Samples of size indicated below:

          - Main Framing Member: 12-inch- (300-mm-) long section with weather stripping, glazing bead and factory-applied color finish.

          - Hardware: Full-size units with factory-applied finish.

          - <Insert component>: <Insert description>.

        - Product Schedule: For multipanel folding wood-framed glass doors.[ Use same designations indicated on Drawings.]

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

For Kolbe's FSC certificate, see https://info.fsc.org/details.php?id=a0240000005sVLmAAM&type=certificate.

          - Chain-of-Custody Certificates: For certified wood products. Include statement of costs.

"Chain-of-Custody Qualification Data" Subparagraph below applies to LEED v4 and LEED v4.1.

          - Chain-of-Custody Qualification Data: For manufacturer and vendor.

      - INFORMATIONAL SUBMITTALS

Informational submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. Retain "testing agency" option if Contractor is providing testing in "Field Quality Control" Article.

        - Qualification Data: For [Installer] [manufacturer] [and] [testing agency].

        - Product Test Reports: For each multipanel folding wood-framed glass door, for tests performed by [manufacturer and witnessed by a qualified testing agency] [a qualified testing agency]; and for each class and performance grade indicated, tested at AAMA gateway size.

        - Field Quality-Control Submittals:

Retain "Field quality-control reports" Subparagraph below if Contractor is responsible for field quality-control testing and inspecting.

          - Field quality-control reports.

        - Sample warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For multipanel folding wood-framed glass doors. Include finishes, weather stripping, operable panels, and operating hardware.

      - QUALITY ASSURANCE

Retain "Certified Wood" Paragraph below for LEED v4 and LEED v4.1, MRc "Building Product Disclosure and Optimization - Sourcing of Raw Materials, Option 2 - Leadership Extraction Practices."

For Kolbe's FSC certificate, see https://info.fsc.org/details.php?id=a0240000005sVLmAAM&type=certificate.

        - Certified Wood: Provide an invoice including vendor's chain-of-custody number, product cost, and entity being invoiced.

Retain "Vendor Qualifications" Paragraph below if required for LEED v4 and LEED v4.1.

        - Vendor Qualifications: A vendor that is certified for chain of custody by an FSC-accredited certification body.

        - Installer Qualifications: An installer acceptable to multipanel folding wood-framed glass door manufacturer for installation of units required for this Project.

      - MOCKUPS

        - Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for fabrication and installation.

          - Build mockup for multipanel folding wood-framed glass doors, as indicated on Drawings.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Owner specifically approves such deviations by Change Order.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - WARRANTY

When warranties are required and available, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Special Warranty: Manufacturer agrees to repair or replace multipanel folding wood-framed glass doors that fail(s) in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for multipanel folding wood-framed glass doors. Manufacturers may offer extended warranties on some components.

          - Warranty Period:

            - Multipanel Folding Wood-Framed Glass Doors: [10] <Insert number> year(s) from date of shipment and issued to Owner at Substantial Completion.

Warranty period for glazing units varies by type. Insulating-glass units typically have longer warranties than laminated glass. Some manufacturers may insist that warranties begin on date of manufacture or sales.

            - Insulating-Glass Units: [10] [20] <Insert number> years from date of shipment and issued to Owner at Substantial Completion.

            - Laminated Glass: [Five] <Insert number> years from date of shipment and issued to Owner at Substantial Completion.

Retain "Aluminum Finish" Subparagraph below for aluminum-clad multipanel folding wood-framed glass doors. Warranty period for aluminum finish depends on type. Baked-on enamel finishes often have 10-year warranties; high-performance fluoropolymer finishes often have 20-year warranties.

            - Aluminum Finish: [20] [30] <Insert number> years from date of shipment and issued to Owner at Substantial Completion.

        - Special Finish Warranty, Factory-Applied Finishes: Manufacturer agrees to repair finishes or replace aluminum that shows evidence of deterioration of factory-applied finishes within specified warranty period.

Coordinate "Warranty Period" Subparagraph below with "Aluminum Finishes" Article. AAMA 2604 is intended to represent five years of performance; AAMA 2605 is intended to represent 10 years of performance. Some manufacturers also offer a 20-year warranty. Verify available warranties and warranty periods for finishes.

          - Warranty Period: [Five] [10] [20] <Insert number> years from date of shipment and issued to Owner at Substantial Completion.

        - Special Finish Warranty, Anodized Finishes: Manufacturer agrees to repair finishes or replace aluminum that shows evidence of deterioration of anodized finishes within specified warranty period.

Coordinate "Warranty Period" Subparagraph below with "Aluminum Finishes" Article. Five years is standard for Class I anodized finishes, although a few manufacturers offer a 10- or 20-year warranty. Class II anodized finishes often carry less than a five-year warranty. Verify available warranties and warranty periods for finishes.

          - Warranty Period: [Five] [10] <Insert number> years from date of shipment and issued to Owner at Substantial Completion.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - MANUFACTURERS

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed.

        - Basis-of-Design Product: Subject to compliance with requirements, provide Kolbe Windows & Doors; [Heritage Series Folding Doors] [Ultra Series Folding Doors] [VistaLuxe Complementary Folding Doors] or comparable product by one of the following:

          - Loewen Windows.

          - Marvin Windows and Doors.

          - <Insert manufacturer's name>.

      - PERFORMANCE REQUIREMENTS

See the Evaluations for discussion of establishing performance requirements using AAMA/WDMA/CSA 101/I.S.2/A440.

        - Product Standard: Comply with AAMA/WDMA/CSA 101/I.S.2/A440 for minimum standards of performance, materials, components, accessories, and fabrication unless more stringent requirements are indicated.

Retain "Product Certification" Subparagraph below if WDMA certification and labeling is needed. Not all manufacturers that test products in accordance with AAMA/WDMA/CSA 101/I.S.2/A440 requirements participate in WDMA's third-party certification program for listing and labeling.

          - Product Certification: WDMA certified with label attached to each door.

Coordinate performance class and grade with available options and features. Some options affect door performance or may not be available for doors meeting selected performance class and grade.

        - Performance Class and Grade: AAMA/WDMA/CSA 101/I.S.2/A440 as follows:

For Kolbe's "Ultra Series," "VistaLuxe Complementary," and "Heritage Series" doors performance class and grade, see www.kolbewindows.com/resources/professional-tools/performance-class-grade.

          - Minimum Performance Class: [Class R] [Class LC] [Class CW] [Class AW] [As indicated on Drawings] <Insert class>.

AAMA/WDMA/CSA 101/I.S.2/A440 establishes a gateway performance grade for multipanel folding wood-framed glass doors to qualify for each performance class. The gateway performance grade is 15 for Class R, 25 for Class LC, 30 for Class CW, and 40 for Class AW. For a particular project, the minimum performance grade for multipanel folding wood-framed glass doors is typically based on the design pressure.

          - Minimum Performance Grade: [Grade 15] [Grade 20] [Grade 25] [Grade 30] [Grade 35] [Grade 40] [Grade 45] [Grade 50] [As indicated on Drawings] <Insert grade>.

Retain "Thermal Transmittance" Paragraph below if required to comply with requirements of authorities having jurisdiction. ENERGY STAR program requirements vary by location; confirm requirements with authorities having jurisdiction. Options listed are examples only.

For Kolbe's folding doors "Energy Performance," see www.kolbewindows.com/resources/professional-tools/energy-performance.

        - Thermal Transmittance: NFRC 100 maximum total fenestration product U-factor of [0.33 Btu/sq. ft. x h x deg F (1.87 W/sq. m x K)] [0.36 Btu/sq. ft. x h x deg F (2.04 W/sq. m x K)] <Insert value>.

Options in "Solar Heat-Gain Coefficient (SHGC)" Paragraph below are based on ENERGY STAR requirements. ENERGY STAR program requirements vary by location; confirm requirements with authorities having jurisdiction. Options listed are examples only.

        - Solar Heat-Gain Coefficient (SHGC): NFRC 200 maximum total fenestration product SHGC of [0.42] [0.40] [0.36] [0.34] <Insert value>.

CRF values are typically between 45 and 75. First option in "Condensation-Resistance Factor (CRF)" Paragraph below is suitable for most of the United States at 25 percent relative humidity, where outdoor design temperature is below 0 deg F (minus 18 deg C); second option is suitable for same relative humidity at minus 10 deg F (minus 23 deg C). In northern states, where winter outdoor design temperature is lower, increase factor. If buildings have mechanically controlled humidification, coordinate CRF with design conditions for humidity level. See AAMA 1503.

        - Condensation-Resistance Factor (CRF): Provide multipanel folding wood-framed glass doors tested for thermal performance in accordance with AAMA 1503, showing a CRF of [45] [52] <Insert value>.

Retain "Windborne-Debris Impact Resistance" Paragraph below to suit Project. The IBC establishes criteria for buildings in hurricane-prone locations. In paragraph, "enhanced" option applies to essential facilities and has additional requirements. Verify requirements of authorities having jurisdiction. Verify which manufacturers have tested products and can demonstrate compliance.

Kolbe's "Ultra Series" folding doors are capable of complying with IPD4 hurricane forces. For more information, see www.kolbewindows.com/solutions/impact-resistance.

        - Windborne-Debris Impact Resistance: Passes ASTM E1886 missile-impact and cyclic-pressure tests in accordance with ASTM E1996 for Wind Zone [1] [2] [3] [4] for [basic] [enhanced] protection.

Insert increased heights if different from those in "Large-Missile Test" and "Small-Missile Test" subparagraphs below. If retaining "enhanced" option in "Windborne-Debris Impact Resistance" Paragraph above, delete "Small-Missile Test" Subparagraph.

          - Large-Missile Test: For glazing located within 30 ft. (9.1 m) <Insert height> of grade.

          - Small-Missile Test: For glazing located between 30 ft. (9.1 m) and [60 ft. (18.3 m)] <Insert height> above grade.

      - MULTIPANEL FOLDING WOOD-FRAMED GLASS DOORS

        - Multipanel Folding Wood-Framed Glass Doors: Provide multipanel folding wood-framed glass doors, designed for exterior use, complete with glazing, threshold, flashings, support, and anchorage devices.

Other configurations are available; verify with manufacturers.

          - Application: [Exterior] [Interior], outward opening.

Panels are normally all hinged together; retain last option in "Stack Storage Configuration" Subparagraph below if panels are hinged together in pairs.

          - Stack Storage Configuration: [Panels open from center to both sides] [As indicated on Drawings] <Insert configuration>.[ Hinge panels together in separate pairs.]

        - Frames and Door Panels: Fabricate from wood components complying with AAMA/WDMA/CSA 101/I.S.2/A440 and indicated requirements. Provide factory-assembled door panels that are reglazable without dismantling panel framing, and factory-assembled frames.

          - Door Panel Design: [Narrow-profile] [Standard-profile] [Wide-profile] <Insert dimensions> design.

Retain "Exterior Aluminum-Clad Exterior Finish" Paragraph below for Kolbe's "Ultra Series" and "VistaLuxe Collection" aluminum-clad wood-framed doors.

Retain first option below for Kolbe's "Ultra Series" or "VistaLuxe WD LINE" standard finish; retain second option for coastal and heavy salt concentration areas.

        - Exterior Aluminum-Clad Exterior Finish: Aluminum cladding with manufacturer's standard [silicon modified polyester (SMP) coating complying with AAMA 2604] [fluoropolymer two-coat system with fluoropolymer color topcoat containing not less than 70 percent PVDF resin by weight and complying with AAMA 2605] [anodized coating] [mica coating complying with AAMA 2605].

          - SMP or PVDF Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Bronze] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coal Black] [Coastal storm] [Copper canyon] [Corbeau] [Emerald isle] [Custom color] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Midnight] [Mystic ivy] [Mudpie] [Nutmeg] [Natural cotton] [Onyx] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

          - Anodized Color: [Black] [Champagne] [Clear] [Dark bronze].

          - Mica Color: [Cosmic gray] [Galaxy silver] [Night sky] [Silverstorm].

Retain "Wood Exterior Finish" Paragraph below for Kolbe's "Heritage Series" unclad wood-framed doors.

        - Wood Exterior Finish: [Manufacturer's standard factory-applied color finish] [Manufacturer's standard factory-applied latex primer] [Manufacturer's standard factory-applied oil primer] [Unfinished].

Retain "Paint Color" Subparagraph below for factory-applied color finish.

          - Paint Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Camel] [Castlerock] [Cloud] [Chutney] [Cider] [Coastal storm] [Copper canyon] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Mudpie] [Mystic ivy] [Natural cotton] [Nutmeg] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White].

Kolbe's standard wood species is "Pine"; custom wood species other than those listed may be requested.

          - Wood Species: [Pine] [Alder] [Bamboo] [Cherry] [Mahogany] [Maple] [Knotty alder] [Mixed grain fir] [Oak] [Sapele] [Spanish cedar] [Vertical grain fir] [Walnut] [White oak]<Insert species>.

        - Interior Surfaces: [Unfinished] [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Manufacturer's standard clear coat] [Manufacturer's standard stain-and varnish-finish].

Retain "Wood Species" Subparagraph below for unfinished or varnished wood.

          - Wood Species: [Pine] [Alder] [Bamboo] [Cherry] [Mahogany] [Knotty alder] [Mixed grain fir] [Oak] [Sapele] [Spanish cedar] [Vertical grain fir] [Walnut] [White oak] <Insert species>.

Retain "Wood Stain Color" or "Paint Color" Subparagraph below for factory-applied color finish.

          - Wood Stain Color: [Cherry] [Chestnut] [Coffee bean] [Double clear coat] [Ebony] [Library red] [Light oak] [Red wheat] [Spiced walnut] [Sunset oak] [Wheat] <Insert color>.

          - Paint Color: [Abalone] [Bright white] [Coal black] [Graystone] [Ivory tusk] [Misty gray] [Natural cotton] [Silk] [Ultra pure white] <Insert color>.

        - Wood Components: Manufacturer's standard LVL or fine-grained wood lumber complying with AAMA/WDMA/CSA 101/I.S.2/A440; kiln-dried to a moisture content of not more than 12 percent at time of fabrication; free of visible finger joints, blue stain, knots, pitch pockets, and surface checks larger than 1/32 inch (0.8 mm) deep by 2 inches (51 mm) wide; water-repellent preservative treated.

First "Certified Wood" Paragraph below applies to LEED v4 and LEED v4.1. Manufacturers certifying products as "FSC Mixed Credit" do not have to use 100 percent certified wood in the products; however, in their total production, manufacturers must use an amount equal to or greater than the percentage of their production that is labeled "FSC Mixed Credit."

For Kolbe's FSC certificate, see https://info.fsc.org/details.php?id=a0240000005sVLmAAM&type=certificate.

        - Certified Wood: Wood doors to be certified as "FSC Pure"[ or "FSC Mixed Credit"] in accordance with FSC STD-01-001 and FSC STD-40-004.

First "Certified Wood" Paragraph below applies to IgCC.

        - Certified Wood: Wood doors to be labeled in accordance with the AF&PA's Sustainable Forestry Initiative, be certified as "FSC Pure" in accordance with FSC STD-01-001 and FSC STD-40-004, or be certified and labeled in accordance with the standards of the Programme for Endorsement of Forest Certification.

First "Certified Wood" Paragraph below applies to ASHRAE 189.1.

        - Certified Wood: Wood doors to [contain not less than 60 percent] [be made from] certified wood tracked through a chain-of-custody process. Certified wood documentation to be provided by sources certified through a forest certification system with principles, criteria, and standards developed using ISO/IEC Guide 59 or the World Trade Organization's "Technical Barriers to Trade."

"Certified Wood" Paragraph below applies to Green Globes.

        - Certified Wood: Wood doors to be certified in accordance with the American Tree Farm System's "AFF Standard," the AF&PA's Sustainable Forestry Initiative, FSC STD-01-001 and FSC STD-40-004, or the standards of the Programme for Endorsement of Forest Certification.

      - GLAZING

Retain second option in "Glass and Glazing" Paragraph below if glass and glazing requirements are indicated in Section 088000 "Glazing." Revise below for custom glass or glazing requirements if available from selected manufacturers.

        - Glass and Glazing: Manufacturer's standard glazing system that produces weathertight seal. [Comply with requirements for windborne-debris resistance] [Comply with requirements indicated in Section 088000 "Glazing"] <Insert glazing requirements>.

Delete "Glass," "Safety Glazing Labeling," "Tint," and "Insulating-Glass Units" subparagraphs below if retaining third option in "Glass and Glazing" Paragraph above.

          - Glass: ASTM C1036, Type 1, q3; Category II safety glass complying with testing requirements in 16 CFR 1201.

Retain "Safety Glazing Labeling" Subparagraph below if applicable and if labeling is required. Not all manufacturers participate in third-party testing programs.

          - Safety Glazing Labeling: Permanently mark safety glazing with certification label of [the SGCC] [the SGCC or another certification agency acceptable to authorities having jurisdiction] [or] [manufacturer]. Label indicates manufacturer's name, type of glass, thickness, and safety glazing standard with which glass complies.

          - Tint: [Clear] [Gray] [Bronze] [Green] <Insert tint>.

Retain option in "Insulating-Glass Units" Subparagraph below after verifying that manufacturers obtain glass units from glass manufacturers that participate in the Insulating Glass Certification Council (IGCC) program. Delete subparagraph below if using monolithic glazing.

          - Insulating-Glass Units: ASTM E2190[, certified through IGCC as complying with requirements of IGCC].

            - Filling: Fill space between glass lites with [air] [argon].

            - Lites: [Two] [Three].

Retain first or second option in "Low-E Coating" Subparagraph below for Kolbe's "Solar Low-E," "Solar Gain Low-E," "Solar Control Low-E," or "Solar Advanced Control Low-E"; retain third option for "ThermaPlus Low-E"; retain fourth option for triple-pane glazing.

            - Low-E Coating: [Pyrolytic on second surface] [Sputtered on second surface] [Sputtered on second or fourth surface] [Sputtered on second or fifth surface].

      - HARDWARE

Nonmagnetic stainless steel, Series 300, or a superior corrosion-resistant-coated metal may be required to meet specific customer or regional needs and for protection against corrosive environments, such as in urban, coastal, or industrial areas.

        - Provide manufacturer's standard hardware, fabricated from a corrosion-resistant material compatible with door panels and other components, and complying with AAMA 907. Provide hardware designed to smoothly operate, tightly close, and securely lock multipanel folding wood-framed glass doors. Size hardware to accommodate panel weights and dimensions. Provide full-perimeter weatherstripping for each door panel.

        - Panel Support System: Provide panel support system designed for size, weight, and performance requirements of multipanel folding wood-framed glass doors indicated. Provide carriers with sealed ball bearings.

          - Overhead Supported: Provide multiwheeled overhead carriers suspended from steel or aluminum track, with lower guide system engaged in threshold for smooth operation. Limit track deflection to no more than [0.10 inch (2.54 mm)] <Insert dimension> between supports when fully loaded.

          - Adjustment: Provide panel support system capable of being adjusted for smooth operation and clearances without needing to remove panels from tracks.

In "Threshold Configuration" Subparagraph below, coordinate threshold type with performance requirements and accessibility requirements. Thresholds of multipanel folding wood-framed glass doors with higher water-infiltration resistance typically have a raised curb, which is not compliant with the United States Access Board's ADA-ABA Accessibility Guidelines; lower thresholds may have lower water-infiltration resistance.

          - Threshold Configuration: Extruded-aluminum [weep sill] [recessed floor track sill][ with concealed drainage system].

Retain first or second option in "Aluminum Finish" Subparagraph below for weep sill; retain second option for recessed floor track sill.

            - Aluminum Finish: [Clear] [Dark bronze] anodized.

Retain "Wood Interior Trim" Subparagraph below if first option in "Threshold Configuration" Subparagraph is retained, and if interior faces of multipanel folding wood-framed glass doors are wood.

            - Wood Interior Trim: Provide wood trim on interior side of threshold, finished to match panel interior.

Hinges that are invisible when panels are closed are available from some manufacturers; revise "Panel Hinges" Paragraph below as required.

        - Panel Hinges: Stainless steel, multileaf hinge with plated finish[ to match exterior]. Provide integral hangers and guides for hinges that engage panel support system.

Revise "Locking System" Paragraph below to suit Project's locking requirements, or insert other requirements.

        - Locking System:

          - Primary Entrance Panel: Provide manufacturer's standard keyed multipoint locking device, with lever handles on the interior and exterior that operate dead bolt and concealed top and bottom rods.

Retain "Secondary Entrance Panel" Subparagraph below if primary entrance is a pair of door panels.

          - Secondary Entrance Panel: Provide [dummy trim on interior and exterior with concealed automatic flush bolts] [two-point locking device that operates concealed top and bottom rods] [surface-mounted flush bolt] <Insert locking type>.

          - Panel Pairs: Provide [two-point locking device that operates concealed top and bottom rods] [surface-mounted flush bolt] at each panel pair.

          - Trim Design: [As selected from manufacturer's full range] <Insert design>.

Finishes indicated in "Finish" Subparagraph below are examples only; confirm availability with manufacturers.

            - Finish: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected from manufacturer's full range of finishes] [Antique brass] [Antique nickel] [Beige] [Brass] [Bright brass] [Clay] [Dull brushed chrome] [Gold painted] [Matte black] [Oil rubbed] [Rustic umber] [Satin nickel] [Smoky gray] [Stainless steel] [White].

In "Cylinders" Subparagraph below, retain first option if manufacturer's standard key-operated cylinders are specified in this Section; retain second option if commercial-grade cylinders are specified in this Section; and retain third or fourth option if cylinders are specified in another Section.

          - Cylinders: [Manufacturer's standard.] [ANSI/BHMA A156.5, Grade 1.] [As specified in Section 087100 "Door Hardware."] [As specified in Section 087111 "Door Hardware (Descriptive Specification)."]

      - ACCESSORIES

Retain "True Divided Lites" or "Grilles (False Muntins)" Paragraph below if required for Project. Some manufacturers have a limited range of options for grilles; confirm availability.

Kolbe's "Heritage Series" offers true divided lites; however, manufacturer range may be limited. For more information, see www.kolbewindows.com/doors/heritage-swinging-patio#Op105&OpT527&OpS2042.

        - True Divided Lites: Provide glazing with true muntins, resulting in individually glazed lites, in pattern indicated.

          - Wood Species: To match adjacent door panel.

          - Design: [Rectangular] [As indicated on Drawings] <Insert design>.

          - Bar Width: Not less than 1-1/8 inches (28 mm) wide.

          - Interior Bar Profile: [Beveled] [Ovolo].

          - Exterior Finish: [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Unfinished].

            - Exterior Paint Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coastal storm] [Copper canyon] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Lunar] [Maize] [Mediterranean] [Mudpie] [Mystic ivy] [Nutmeg] [Khaki] [Natural cotton] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

          - Interior Finish: [Unfinished] [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Manufacturer's standard clear coat] [Manufacturer's standard stain-and varnish-finish].

            - Interior Stain Color: [Cherry] [Chestnut] [Coffee bean] [Double clear coat] [Ebony] [Library red] [Light oak] [Red wheat] [Spiced walnut] [Sunset oak] [Unfinished pine] [Wheat] <Insert color>.

            - Interior Paint Color: [Abalone] [Bright white] [Coal black] [Graystone] [Ivory tusk] [Misty gray] [Natural cotton] [Silk] [Ultra pure white] <Insert color>.

        - Grilles (False Muntins): Provide grilles in designs indicated to each panel lite.

          - Type: Exterior and interior, adhered to glass with bars between the glass.

          - Material: To match adjacent material and finish of window panel.

          - Design: [Rectangular] [Diamond] [As indicated on Drawings] <Insert design>.

          - Bar Width: Not less than [5/8 inch (16 mm)] [7/8 inch (22 mm)] [1-1/8 inches (28 mm)] [1-3/8 inches (35 mm)] [1-3/4 inches (44.5 mm)] [2-1/4 inches (28 mm)] [4-1/2 inches (114 mm)] wide.

Retain one or more of first three options for Kolbe's "Heritage Series" or "Ultra Series"; retain third or fourth option for Kolbe's "VistaLuxe Collection."

          - Bar Profile: [Beveled] [Ovolo] [Square] [Triangular].

Retain "Exterior Aluminum Finish" or "Exterior Wood Finish" Subparagraph below.

Retain first option in "Exterior Aluminum Finish" Paragraph below for Kolbe's "Ultra Series" or "VistaLuxe WD LINE" standard finish; retain second option for coastal and heavy salt concentration areas.

          - Exterior Aluminum Finish: Aluminum cladding with manufacturer's standard [silicon modified polyester (SMP) coating complying with AAMA 2604] [fluoropolymer two-coat system with fluoropolymer color topcoat containing not less than 70 percent PVDF resin by weight and complying with AAMA 2605] [anodized coating] [mica coating complying with AAMA 2605].

            - SMP or PVDF Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Bronze] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coal black] [Coastal storm] [Copper canyon] [Corbeau] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Midnight] [Mystic ivy] [Mudpie] [Nutmeg] [Natural cotton] [Onyx] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

            - Anodized Color: [Black] [Champagne] [Clear] [Dark bronze].

            - Mica Color: [Cosmic gray] [Galaxy silver] [Night sky] [Silverstorm].

          - Exterior Wood Finish: [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Unfinished].

            - Exterior Paint Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coastal storm] [Copper canyon] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Lunar] [Maize] [Mediterranean] [Mudpie] [Mystic ivy] [Nutmeg] [Khaki] [Natural cotton] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

          - Interior Finish: [Unfinished] [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Manufacturer's standard clear coat] [Manufacturer's standard stain-and varnish-finish].

            - Interior Stain Color: [Cherry] [Chestnut] [Coffee bean] [Double clear coat] [Ebony] [Library red] [Light oak] [Red wheat] [Spiced walnut] [Sunset oak] [Wheat].

            - Interior Paint Color: [Abalone] [Bright white] [Coal black] [Graystone] [Ivory tusk] [Misty gray] [Natural cotton] [Silk] [Ultra pure white].

        - Casing: Extruded-aluminum [1-5/8-inch (41-mm) brickmould] [1-15/16-inch (49-mm) brickmould] [2-inch (51-mm) flat casing] [2-inch (51-mm) flat casing with backband] [3-1/2-inch (89-mm) flat casing] [3-1/2-inch (89-mm) flat casing with backband] [3-1/2-inch (89-mm) profiled brickmould] [beaded casing] [beaded casing with backband] [bullnose casing] [historic brickmould] [ogee casing] [pavilion brickmould] [stucco trim casing].

        - Casing: Exterior wood [bullnose casing] [prospect casing] [1-15/16-inch (49-mm) flat casing] [1-15/16-inch (49-mm) brickmould] [2-inch (51-mm) beaded casing] [1-15/16-inch (49-mm) flat casing with backband (one piece)] [3-1/2-inch (89-mm) profiled brickmould] [3-1/2-inch (89-mm) beaded casing] [3-1/2-inch (89-mm) flat casing] [3-1/2-inch (89-mm) profiled brickmould with backband] [3-1/2-inch (89-mm) flat casing with backband] [4-inch (102-mm) flat casing] [5-inch (127-mm) flat casing].

        - Fasteners: Noncorrosive and compatible with door members, trim, hardware, anchors, and other components.

Generally, retain "Exposed Fasteners" Subparagraph below. Revise if exposed fasteners are permitted.

          - Exposed Fasteners: Do not use exposed fasteners to greatest extent possible. For application of hardware, use fasteners that match finish hardware being fastened.

        - Anchors, Clips, and Accessories: Provide anchors, clips, and accessories of aluminum, nonmagnetic stainless steel, or zinc-coated steel or iron for multipanel folding wood-framed glass doors, complying with ASTM B456 or ASTM B633 for SC 3 severe service conditions; provide sufficient strength to withstand design pressure indicated.

          - Windborne-Debris Resistance: Provide anchors of same design used in windborne-debris resistance testing.

      - INSECT SCREENS

        - Screen Fabric: Provide screen mesh of PVC-coated, glass-fiber threads; woven and fused to form a fabric mesh resistant to corrosion, shrinkage, stretch, impact damage, and weather deterioration; and complying with ASTM D3656/D3656M.

          - Fabric Color: [Manufacturer's standard] [Charcoal gray] [Silver gray] <Insert color>.

        - Pleated Insect Screen: Screen suspended from the side, with bottom guide, and finish in manufacturers' standard black mesh. Provide single- or two-function operation with screen meeting rails at same location as primary entrance of door.

      - FABRICATION

        - Complete fabrication, assembly, finishing, hardware application, and other work in the factory to greatest extent possible. Disassemble components only as necessary for shipment and installation.

        - Factory-Glazed Fabrication: Glaze multipanel folding wood-framed glass doors in the factory.

      - GENERAL FINISH REQUIREMENTS

        - Appearance of Finished Work: Noticeable variations in same piece are unacceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - ALUMINUM FINISHES

Retain finishes in paragraphs below to suit Project. If retaining more than one, indicate location of each on Drawings or by inserts.

        - Silicon Modified Polyester (SMP): AAMA 2604; epoxy primer and silicone-modified, polyester-enamel topcoat; with a dry film thickness of not less than 0.2 mil (0.005 mm) for primer and 0.8 mil (0.02 mm) for topcoat.

Retain "Basis-of-Design Product" Subparagraph below to identify a specific product or a comparable product.

For Kolbe's "Vantage Exterior SMP Finish" catalogue, see www.kolbewindows.com/resources/brochures.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Kolbe Windows & Doors; Vantage Exterior SMP Finish or comparable product.

        - High-Performance Organic Finish, Two-Coat PVDF: Fluoropolymer finish complying with AAMA 2605 and containing not less than 70 percent PVDF resin by weight in color coat.

        - Mica Fluoropolymer: AAMA 2605. Two-coat fluoropolymer finish with suspended mica flakes containing not less than 70 percent PVDF resin by weight in color coat. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

        - Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44, Class I, 0.018 mm] [AA-M12C22A32/A34, Class II, 0.010 mm] or thicker.

      - WOOD FINISHES

Retain this article if wood multipanel folding wood-framed doors are provided with factory finishes on the interior or exterior surfaces.

        - Factory-Applied Primer: Provide manufacturer's standard factory-applied prime coat complying with WDMA T.M. 11. <Insert specific requirements for factory-applied prime coat if any> on exposed [exterior] [and] [interior] wood surfaces.

First option in "Factory-Applied Color Finish" Paragraph below is for pigmented (paint) coatings suitable for exterior exposure.

        - Factory-Applied Color Finish: Provide manufacturer's standard factory-applied finish [complying with WDMA T.M. 12] <Insert manufacturer's standard finish>.

- EXECUTION

      - EXAMINATION

        - Examine openings, substrates, structural support, anchorage, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of Work.

        - Verify rough opening dimensions, levelness of threshold substrate, and operational clearances.

        - Examine wall flashings, vapor retarders, water and weather barriers, and other built-in components to ensure a coordinated, weathertight hinged door installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Comply with Drawings, Shop Drawings, and manufacturer's written instructions for installing multipanel folding wood-framed glass doors, hardware, accessories, and other components.

Retain "Windborne-Debris Resistance" Paragraph below for installations requiring windborne-debris resistance. Proper anchorage of units is necessary to maintain performance as tested.

        - Windborne-Debris Resistance: Anchor multipanel folding wood-framed glass doors that have been tested for windborne-debris resistance to structure, using anchoring method, fastener type, and fastening frequency identical to that used in windborne-debris resistance testing.

        - Install multipanel folding wood-framed glass doors level, plumb, square, true to line; without distortion, warp, or rack of frames and panels, and without impeding thermal movement; anchored securely in place to structural support; and in proper relation to wall flashing, vapor retarders, air barriers, water/weather barriers, and other adjacent construction.

        - Set threshold members in bed of sealant or with gaskets, as indicated, to provide weathertight construction.

        - Separate aluminum and other corrodible surfaces from sources of corrosion or electrolytic action at points of contact with other materials.

      - FIELD QUALITY CONTROL

Retain this article if field tests are required. If retaining, indicate number of doors to be tested. Field tests for air and water leakage should be specified for significant projects.

Retain first option in "Testing Agency" Paragraph below if Owner hires an independent testing agency.

        - Testing Agency: [Owner will engage] [Engage] a qualified testing agency to perform tests and inspections.

          - Testing and inspecting agency will interpret tests and state in each report whether tested work complies with or deviates from requirements.

Revise "Testing Services" Paragraph below if more stringent testing is required. AAMA 502 Test Method A is default unless otherwise indicated.

        - Testing Services: Test and inspect installed multipanel folding wood-framed glass doors as follows:

          - Testing Methodology: Test multipanel folding wood-framed glass doors for air infiltration and water resistance in accordance with AAMA 502.

          - Air-Infiltration Testing:

            - Test Pressure: That required to determine compliance with AAMA/WDMA/CSA 101/I.S.2/A440 performance class indicated.

            - Allowable Air-Leakage Rate: [1.5] <Insert number> times the applicable AAMA/WDMA/CSA 101/I.S.2/A440 rate for product type and performance class rounded down to one decimal place.

          - Water-Resistance Testing:

            - Test Pressure: [Two-thirds] <Insert number> times test pressure required to determine compliance with AAMA/WDMA/CSA 101/I.S.2/A440 performance grade indicated.

            - Allowable Water Infiltration: No water penetration through the unit.

          - Testing Extent: [Three] [Three mockup] <Insert number or description> multipanel folding wood-framed glass doors of each type as selected by Architect and a qualified independent testing and inspecting agency. Conduct tests after perimeter sealants have cured.

          - Test Reports: Prepared in accordance with AAMA 502.

See Section 014000 "Quality Requirements" for retesting and reinspecting requirements and Section 017300 "Execution" for requirements for correcting the Work.

        - Multipanel folding wood-framed glass door will be considered defective if it does not pass tests and inspections.

      - ADJUSTING

        - Adjust hardware for proper alignment, smooth operation, and proper latching without unnecessary force or excessive clearance.

        - Adjust hardware and operable panels to function smoothly, and lubricate as recommended by manufacturer.

      - CLEANING

        - Clean exposed surfaces immediately after installation. Avoid damaging protective coatings and finishes. Remove nonpermanent labels, excess sealants, glazing materials, dirt, and other substances.

        - Remove and replace glass that has been broken, chipped, cracked, abraded, or damaged during construction period.

      - PROTECTION

        - Protect multipanel folding wood-framed glass door surfaces from contact with contaminating substances resulting from construction operations. Remove contaminants immediately in accordance with manufacturer's written instructions.

Retain paragraph below if multipanel folding wood-framed glass doors are factory finished.

        - Refinish or replace folding wood-framed glass doors with damaged finishes.

END OF SECTION 083519.13